General Questions

- What are the four layers of the Basic Software (BSW)?

  - Service Layer

  - ECU Abstraction Layer

  - Microcontroller Abstraction Layer (MCAL)

  - Complex Device Drivers

- Which layer is the uppermost layer in the BSW?

  - The Service Layer

Service Layer

- What is the main task of the Service Layer?

  - Providing basic services to the application and basic software modules.

- Name at least three types of services provided by the Service Layer.

  - Operating system services

  - Vehicle network communication and management services

  - Memory services (NVRAM management)

  - Diagnostic services (including UDS communication and error memory)

  - ECU state management

- Which communication standard is supported by the diagnostic services in the Service Layer?

  - UDS (Unified Diagnostic Services) communication

- What kind of memory management is handled in the Service Layer?

  - NVRAM (Non-Volatile RAM) management

- List three functionalities of the Service Layer.

  - Diagnostics

  - NVRAM Management

  - Watchdog Manager

  - Communication

  - OS (Operating System)

  - Schedule Manager

  - ECU state management

  - Com Channel Management

- How does the Service Layer support the application and other software modules?

  - By providing basic system services needed for communication, memory management, diagnostics, and ECU state management.

ECU Abstraction Layer

- What is the main function of the ECU Abstraction Layer?

  - Providing an API for devices and organizing I/O hardware access, making higher software layers independent from ECU hardware.

- How does the ECU Abstraction Layer help in hardware abstraction?

  - It abstracts the internal and external periphery devices by providing a uniform interface independent of the actual hardware location.

- What does the term “periphery” refer to in the context of this layer?

  - Periphery refers to the internal and external hardware components and interfaces (e.g., sensors, actuators, I/O devices) connected to the ECU.

- Why is the location of devices (internal or external) irrelevant in the ECU Abstraction Layer?

  - Because the ECU Abstraction Layer provides an interface that hides the physical location of devices, allowing software to access devices uniformly regardless of whether they are internal to the microcontroller or external.

- How does the ECU Abstraction Layer support API independence from the underlying hardware?

  - By offering standardized APIs that do not depend on device location or specific hardware details, enabling higher layers to function without hardware-specific code.

- What is the main task of the ECU Abstraction Layer according to the text?

  - Making higher software levels independent of the ECU hardware.

General Questions

- What does MCAL stand for?

  - Microcontroller Abstraction Layer

- Which layer is the lowermost in the Basic Software (BSW) architecture?

  - The Microcontroller Abstraction Layer (MCAL)

- Which two components are described in this section of the Layered Software Architecture?

  - Microcontroller Abstraction Layer (MCAL)

  - Complex Device Drivers

MCAL (Microcontroller Abstraction Layer)

- What is the main task of the MCAL?

  - Making higher layers independent of the microcontroller.

- What kind of hardware does the MCAL have direct access to?

  - Microcontrollers, their internal peripherals, and memory-mapped microcontrollers of external devices.

- How does the MCAL contribute to hardware abstraction in AUTOSAR?

  - By providing drivers that access the microcontroller and its peripherals directly, MCAL abstracts microcontroller-specific details from higher software layers.

- Why is it important for higher software layers to be independent of the microcontroller?

  - To enable portability and scalability of software across different microcontrollers without modifying higher-level software components.

Complex Device Drivers

- What makes Complex Device Drivers a special case in AUTOSAR’s layered architecture?

  - They have direct access to microcontroller resources and handle special sensors and actuators that require unique timing or non-AUTOSAR technology integration.

- What is the primary task of Complex Device Drivers?

  - Controlling special sensors and actuators by direct microcontroller access, especially when subject to special timing conditions or non-standard technology integration.

- Give examples of situations where Complex Device Drivers are used.

  - Injection control

  - Battery management system

- How do Complex Device Drivers handle special timing conditions?

  - By accessing the microcontroller directly to meet precise timing requirements that cannot be handled by standard abstraction layers.

- What kind of hardware resources can Complex Device Drivers access directly?

  - Microcontroller resources, including hardware peripherals needed for critical applications.

- Why might a system need to integrate non-AUTOSAR technologies using Complex Device Drivers?

  - Because some sensors or actuators use technologies or timing conditions that are not covered by standard AUTOSAR modules and thus require custom handling at a lower level.

COM Module

- What is the main function of the COM module?

  - It provides a signal-based data interface for the RTE, placing signals into messages and sending them according to the defined send type.

- How does the COM module handle signals before sending them?

  - It places signals into messages and sends them according to the defined send type.

- What mechanisms does the COM module use for receiving signals?

  - It contains various notification mechanisms for receiving signals.

- Which elements does the COM module manage at the signal level?

  - Initial values, update bits, and timeouts.

- How does the COM module support multi-channel ECUs?

  - Through an integrated signal gateway that can route signals between communication buses.

IPDU Multiplexer

- What is the purpose of the IPDU Multiplexer module?

  - It supports multiple layouts that can be chosen by a selector field within an I-PDU.

- How does the IPDU Multiplexer allow for multiple layouts within an I-PDU?

  - By using a selector field within the I-PDU to select the layout.

- What alternative feature does the IPDU Multiplexer provide for handling PDUs?

  - It supports dynamically combining PDUs into a Container PDU.

PDU Router (PduR)

- What is the role of the PDU Router in the AUTOSAR architecture?

  - It handles distribution of communication packets (PDUs) between bus systems and various BSW modules.

- What types of routing mechanisms does PduR support for IF-PDUs?

  - Last-is-best or FIFO routing.

- What types of routing mechanisms does PduR support for TP-PDUs?

  - Store-and-forward or on-the-fly routing.

- How does the PduR facilitate communication between different bus systems?

  - By providing gateway mechanisms that route PDUs between bus systems.

Transport Protocol

- What type of data transmission does the Transport Protocol handle?

  - Segmented data transmission.

Interface

- What are the key features of the Interface layer?

  - Hardware independent, bus-typical functions including queuing, assembly of data link layer PDUs/frames, and Tx/Rx triggering on time-triggered buses.

- How does the Interface layer support bus-specific functions independently of hardware?

  - By providing generic bus-typical functions abstracted from the actual hardware.

- What are the main tasks performed by the Interface?

  - Queuing data, assembling data link layer PDUs/frames, and triggering transmission and reception on time-triggered buses.

Driver

- What is the main responsibility of the Driver layer?

  - Performing hardware operations such as initialization and buffer management.

- Which hardware-related tasks are performed by the Driver?

  - Initialization, filling of buffers, and handling interrupts via the Interrupt Service Routine (ISR).

- What is the role of the Interrupt Service Routine (ISR) in the Driver?

  - To handle hardware interrupts efficiently for communication and other events.

Transceiver

- What is the primary function of the Transceiver?

  - Operation of hardware line driver circuits.

- What kind of hardware does the Transceiver operate?

  - Hardware line driver circuits that manage physical bus signals.

ECUM – ECU State Manager

- What does EcuM stand for?

  - ECU State Manager.

- What fundamental ECU states are managed by EcuM?

  - On, Off, and Sleep.

- What is the role of EcuM in state transitions?

  - It performs the necessary transitions between ECU states.

- How does EcuM handle wake-up events?

  - It centralizes and validates wake-up events.

- What is the first function you call to initialize an AUTOSAR stack?

  - EcuM_init.

- List the main responsibilities of EcuM during ECU operation.

  - Initial startup of the system, starting the OS, proper shutdown of the system, transitioning the system into sleep mode.

- During what phases of the system lifecycle is EcuM active (e.g., startup, shutdown)?

  - During startup, normal operation, shutdown, and sleep mode transitions.

COMM – Communication Manager

- What does ComM stand for?

  - Communication Manager.

- What kind of state machine is implemented in ComM?

  - An abstracted bus state machine.

- What is ComM responsible for in terms of communication channels?

  - Controlling the communication states of all channels, including startup and shutdown of communication.

- Which modules does ComM use to change the states of communication buses?

  - Bus specific modules called Bus State Manager (Bus SM).

- How is ComM controlled or influenced?

  - Controlled by the application from the top and influenced externally through network management.

- Why is ComM considered independent of hardware and technology?

  - Because it operates at an abstracted level, independent of the underlying hardware and communication technology.

- What role does ComM play as an interface to the application?

  - It serves as the direct interface where the application can request or control communication.

- Who or what can trigger communication through ComM?

  - The application and external influences via network management can trigger communication.

NM IF and Bus NM (Network Management Interface and Bus Network Management)

- What do NM If and Bus NM stand for?

  - NM If: Network Management Interface

  - Bus NM: Bus Network Management

- What is the main responsibility of the NM modules?

  - Handling network management, including keeping the bus awake or coordinating shutdown.

- How do NM If and Bus NM contribute to network wake-up and shutdown?

  - They keep the bus awake or coordinate its shutdown based on network conditions.

- What do these modules do when the ECU is in normal mode or the bus segment is awake?

  - They send periodic messages to maintain bus activity.

- How is busload reduced in a segment using NM?

  - Only two ECUs in the bus segment send NM messages while all others listen, reducing message traffic.

- According to the example, how does NM interact with ComM when an external request from the bus is received?

  - NM wakes ComM and notifies it about the external request so that ComM can wake other ECUs.

- How does the application layer communicate the desire to keep the bus awake?

  - The application requests ComM to keep the bus awake.

- What is the role of ComM in this scenario?

  - ComM communicates with NM to ensure the bus remains awake as requested by the application.

Bus SM (Bus State Manager)

- What is the role of the Bus State Manager?

  - To switch the states of the transceiver and controller hardware.

- What hardware components' states are switched by Bus SM?

  - Transceiver and controller states.

- How does Bus SM interpret requests from ComM?

  - It translates network communication mode requests received from ComM.

- What does Bus SM translate network mode requests into?

  - Changes in the state of the transceiver and controller.

Service Ports (Integration Concept)

- What is a service port in the context of AUTOSAR modules?

  - A mechanism through which modules expose their services directly to the application or other components.

- Which modules typically provide service ports to the application?

  - EcuM (ECU State Manager), ComM (Communication Manager), BswM (Basic Software Manager).

- How does the use of service ports benefit the integration of modules like EcuM, ComM, and BswM?

  - It allows direct connection and interaction between the application and these modules as if they were standard components, simplifying communication and integration.

WdgM – Watchdog Manager

- What is the main function of the Watchdog Manager (WdgM)?

  - To manage alive indications from all supervised entities (such as application runnables) and supervise the execution of software.

- What are alive indications in the context of WdgM?

  - Signals or notifications sent by supervised runnables to inform the Watchdog Manager that they are operating correctly.

- What kind of entities does WdgM supervise?

  - Runnables of the application software.

- What happens when runnables behave as expected?

  - WdgM triggers the Watchdog Interface (WdgIf) to reset the watchdog timer, indicating everything is fine.

- What does WdgM derive based on the status of the supervised entities?

  - It derives a Global Supervision Status.

- How does WdgM interact with the application?

  - It offers interfaces for the application to register runnables and to supervise their execution status.

- What types of supervision can be implemented with WdgM (as per the example)?

  - Deadline monitoring, live supervision, and other execution checks.

- What action does WdgM take if a Runnable fails to check in on time?

  - It cannot inform the Watchdog Driver, which eventually leads to the watchdog timer expiring and a system reset.

WdgIf – Watchdog Interface

- What is the function of the Watchdog Interface (WdgIf)?

  - To trigger hardware watchdog(s) based on signals from the Watchdog Manager.

- When is WdgIf triggered?

  - When the WdgM confirms that all supervised runnables behave correctly and resets the watchdog timer.

WdgDrv – Watchdog Driver

- What does the Watchdog Driver (WdgDrv) do?

  - Controls the actual watchdog hardware, either internal or external, by managing its timer.

- What types of watchdogs can WdgDrv control?

  - Both internal (on-chip) and external watchdog hardware.

Supervision Workflow

- How do runnables register with the WdgM?

  - Runnables register themselves with the Watchdog Manager to be supervised.

- What must runnables do to indicate they are running correctly?

  - They must regularly inform or "check in" with the Watchdog Manager within a certain time frame.

- What happens if not all registered runnables report to the WdgM in time?

  - WdgM is unable to notify the Watchdog Driver that the system is healthy.

- What is the final consequence if the WdgM does not inform the watchdog driver?

  - The watchdog timer expires, leading to a restart of the whole ECU.

Memory Services Cluster – Overview

- When are Memory Services used in an AUTOSAR system?

  - When something needs to be stored into non-volatile memory.

- What types of memory do the Memory Services abstract?

  - Internal (on-chip) and external (on-board) memory devices.

- What are the three main layers that the Memory Services are subdivided into?

  - Service Module (NVRAM Manager), ECU Abstraction (Memory Abstraction Interface, EEPROM Abstraction, Flash EEPROM Emulation), and MCAL Layer (Memory Drivers).

- Which module belongs to the Service Layer?

  - NVRAM Manager.

- Which modules belong to the ECU Abstraction Layer?

  - Memory Abstraction Interface, EEPROM Abstraction, and Flash EEPROM Emulation.

- Which components belong to the MCAL Layer?

  - Memory Drivers (hardware-specific drivers).

NVRAM Manager

- What is the role of the NVRAM Manager?

  - It provides the interface to the application for accessing non-volatile memory.

- How does the application interface with non-volatile memory?

  - Through sender/receiver and client/server ports provided by the NVRAM Manager.

- How does the NVRAM Manager allow writing to memory?

  - It allows writing of configured memory blocks to the lower memory layers.

- What does AUTOSAR use instead of files for storing data?

  - Memory blocks (blocks of data of certain predefined lengths).

- Why is dynamic memory allocation not possible in this context?

  - Because memory blocks are fixed and must be configured in advance.

Memory Abstraction Interface

- What is the purpose of the Memory Abstraction Interface?

  - To map APIs to the lower-layer abstraction modules.

- How does it interact with lower-layer modules?

  - It acts as an intermediary, forwarding requests to EEPROM abstraction or Flash EEPROM emulation modules.

EEPROM Abstraction & Flash EEPROM Emulation

- What does the EEPROM abstraction & Flash EEPROM emulation module handle?

  - Technology-specific abstraction of non-volatile memory access.

- What two technologies are specifically supported in AUTOSAR for non-volatile memory?

  - EEPROM and Flash Memory.

- What do these abstraction modules provide in terms of memory access?

  - Different approaches to reading and writing depending on the underlying technology.

- Why are there different approaches for reading and writing memory in these modules?

  - Because EEPROM and Flash memories have different physical characteristics and access methods.

Hardware-Specific Memory Drivers

- Where are hardware-specific drivers used?

  - For external memory blocks and internal Flash or EEPROM memory.

- What types of memory do these drivers typically handle?

  - On-chip internal Flash or EEPROM and external memory devices.

- At what level (internal/external) are these drivers operating?

  - Both internal (on-chip) and external (on-board) memory devices.

Function Inhibition Manager (FIM)

- What does the Function Inhibition Manager (FIM) control?

  - It controls the conditional execution of application functionality.

- On what condition does FIM execute application functionality?

  - Depending on the contents of the error memory (diagnostic events).

- From where does FIM get the current failure status?

  - From the Diagnostic Event Manager (DEM).

- What does FIM set for a configurable number of diagnostic events?

  - The inhibition status.

- How does the application check the inhibition status in FIM?

  - The inhibition status is polled by the application using the function Fim_GetFunctionPermission.

Diagnostic Event Manager (DEM)

- What is the primary role of the Diagnostic Event Manager?

  - To save diagnostic events in the error memory.

- What type of data does DEM save?

  - Diagnostic events.

- From which sources does DEM collect diagnostic events?

  - From Basic Software (BSW) modules and the Application.

Diagnostic Communication Manager (DCM)

- What protocol does the Diagnostic Communication Manager implement?

  - The diagnostic communication protocol.

- Is the DCM dependent on any specific bus system?

  - No, it is bus independent.

- Why is bus independence important for the DCM?

  - It allows DCM to function across different communication buses without being tied to a specific bus technology.

IoHwAb – I/O Hardware Abstraction

- What is the main purpose of the IoHwAb module?

  - To represent the connection between the RTE and the ECU’s I/O channels by abstracting access to I/O drivers.

- Which two entities does IoHwAb connect in the AUTOSAR architecture?

  - It connects the RTE (Runtime Environment) and the ECU’s I/O peripheral devices.

- What kind of devices does IoHwAb abstract?

  - The location and layout of I/O peripheral devices, such as on-chip or on-board devices.

- Does IoHwAb abstract sensors and actuators directly?

  - No, it does not abstract sensors and actuators directly.

- What kind of signal interface does IoHwAb provide?

  - It provides an I/O signal interface.

- What kind of signal processing does IoHwAb perform on inputs?

  - Conditioning of the read input values.

- Give an example of a signal conditioning task done by IoHwAb.

  - Debouncing of signals.

- Which drivers are encapsulated by the IoHwAb module?

  - ADC (Analog-to-Digital Converter), DIO (Digital Input/Output), PWM (Pulse Width Modulation) drivers.

AUTOSAR OS Module

- What standard is the AUTOSAR OS based on?

  - The OSEK/VDX operating system standard.

- How is the AUTOSAR OS related to the RTE?

  - The OS works closely with the RTE; both must be regenerated if SWCs change.

- Why must the OS and RTE be regenerated if a software component (SWC) changes?

  - Because changes (e.g., runnable timing) affect both OS alarms/timers and RTE event handling.

- Give an example scenario where both the OS and RTE must be regenerated.

  - When the periodical event of a runnable is changed, the RTE updates alarm settings, and the OS updates timers accordingly.

- Why are both the OS and RTE considered static?

  - Because they must be regenerated to reflect any changes and do not adapt dynamically at runtime.

- What are the main elements of the AUTOSAR OS?

  - Tasks, Events, Alarms, Interrupts, Inter-OS Communication (IOC), and Resources.

- What does IOC stand for in the context of the OS?

  - Inter-OS Communication.

- What are the Scalability Classes (SC) used for in AUTOSAR OS?

  - They define different functional extensions of the OS with increasing complexity (SC1 to SC4).

- Which functionalities are covered by SC1?

  - Schedule tables.

- Which additional features are introduced in SC2 compared to SC1?

  - Timing protection plus schedule tables.

- What extra protection does SC3 provide beyond SC2?

  - Memory protection in addition to timing protection and schedule tables.

- What combination of features does SC4 provide?

  - Memory protection, timing protection, and schedule tables.