Runtime Environment in Relation to Runnables

- What is the function of the RTE in relation to runnables?

  - It configures itself and parts of the OS, triggers runnables via events, handles inter-component communication, and abstracts the OS from the SWCs.

- In what way does the RTE configure itself regarding runnables?

  - It maps runnables to OS tasks.

- What parts of the Operating System does the RTE configure?

  - Tasks, events, and alarms.

- How does the RTE handle communication between components?

  - It implements the communication via generated interface code.

- What does the RTE abstract to prevent SWCs from directly accessing?

  - The Operating System and Basic Software (BSW).

- Which elements of the RTE are of existential importance to runnables?

  - The executable code units that call the runnables.

- Why are executable code units with calls of the runnables important?

  - They are needed for the runnables to be triggered and executed.

RTE Events Which Trigger Runnables

- What kind of system is the RTE in relation to how it activates runnables?

  - An event-driven system.

- What must occur to activate a runnable in the RTE?

  - An RTE event.

- Name three examples of events that cause the RTE to execute a particular runnable.

  - Init, TimingEvent, OperationInvokedEvent.

- What is a DataReceivedEvent and in what context does it occur?

  - It’s an event triggered when data is received in Sender/Receiver (S/R) communication.

- What is an OperationInvokedEvent and to which communication type does it belong?

  - It indicates that a client has invoked a server operation in Client/Server (C/S) communication.

- What new events were introduced in AUTOSAR 4.x?

  - ExternalTriggerOccurredEvent, InternalTriggerOccurredEvent, BackgroundEvent.

- What is the difference between ExternalTriggerOccurredEvent and InternalTriggerOccurredEvent?

  - External is triggered by a different SWC; internal is triggered within the same SWC.

- What is a BackgroundEvent used for?

  - For low-priority triggering when no other event has triggered a runnable.

How Does the Syntax Look Like?

- What is the syntax of a runnable triggered by any event except communication events?

  - void runnable (Rte_Instance self)

- What is the syntax of a server runnable triggered by an OperationInvokedEvent with two IN parameters and one OUT parameter?

  - void LightServer (Rte_Instance self, UInt8 keyValue, Boolean active, ResultTypeRef result)

RTE as a Communication Interface

- What does the RTE act as in terms of communication?

  - A communication interface and implementation of the Virtual Functional Bus (VFB).

- What is the purpose of the VFB in the context of RTE?

  - To provide a high level of abstraction, making the code independent of hardware and BSW details.

Inter ECU Sender/Receiver Communication

- In the case of communication between two SWCs on different ECUs, which functions are used by the Runnables?

  - Rte_Write_Port_Data() and Rte_Read_Port_Data()

- What remains the same in the code regardless of whether communication happens between the Runnable and BSW or between two ECUs?

  - The RTE API code.

- Which part changes in the BSW layer depending on communication direction?

  - The Com_SendSignal() and Com_ReceiveSignal() implementation.

- Why is the code considered highly portable and reusable in AUTOSAR?

  - Because it is independent of the communication mechanism and BSW details.

Sender/Receiver Communication

- What are the three types of Sender/Receiver communication in AUTOSAR?

  - Direct, Buffered, and Queued communication.

- What kind of communication allows data exchange such as a sensor sending velocity values?

- Sender/Receiver communication.

- What communication directions are supported by all types of Sender/Receiver communication?

- 1:n (one sender, many receivers) and n:1 (many senders, one receiver).

Direct Communication

- In direct communication, where does a runnable write its data?

- Into a buffer provided by the RTE.

- What is the principle used in direct communication for handling multiple writes?

- Last-is-best: the last write overwrites previous data.

- What data remains in the buffer in direct communication?

- Only the data from the most recent write.

Buffered Communication

- What concept is implemented in buffered communication?

- A shadow buffer.

- What happens to the shadow buffer during a runnable's execution?

- The runnable writes all its data into the shadow buffer.

- When is data copied from the shadow buffer to the actual buffer?

- After the runnable finishes execution.

Queued Communication

- What kind of data structure does queued communication use?

- A FIFO queue.

- How does queued communication differ from direct communication in terms of semantics?

- It uses queued semantics rather than last-is-best.

- What happens on each write and read operation in queued communication?

- Each write adds a new element to the queue; each read removes one.

Exclusive Areas

- Why is it normal for the OS to interrupt a running task within the same software component?

  - Because runnables in the same SWC can run on different OS tasks.

Inter Runnable Variables (IRVs)

- What is the purpose of an Inter Runnable Variable (IRV)?

  - To enable communication between runnables in the same SWC.

- Which component provides an interrupt-safe access API for IRVs?

  - The RTE.

Exclusive Areas (EAs)

- What is the function of Exclusive Areas?

  - To define atomic sections for synchronized access to shared resources.

- What are some examples of configurable implementations for Exclusive Areas?

  - Interrupt locking, OS resource management.

Client/Server Communication

- What is the direction of communication in Client/Server communication?

  - n:1 (multiple clients to one server).

- How does Client/Server communication differ from Sender/Receiver communication?

  - It involves invoking operations (services), not exchanging data.

- What is an example of a service a server might provide?

  - Starting an engine.

AUTOSAR Interfaces

- What are the three types of AUTOSAR interfaces?

  - AUTOSAR interface, Standardized interface, Standardized AUTOSAR interface.

- What defines a Standardized interface in AUTOSAR?

  - Its semantics are prescribed by the AUTOSAR standard.

- Give an example of a function from a standardized interface.

  - Rte_Start() or Rte_Stop() from EcuM.

- What defines a generic AUTOSAR interface?

  - It's described using AUTOSAR mechanisms, but its semantics are not prescribed by AUTOSAR.

- Where are generic AUTOSAR interfaces used?

  - By SWCs and ECU Abstraction.

- What is the API function format for Sender/Receiver communication?

  - Rte_Write_<p>_<d>()

- What is the API function format for Client/Server communication?

  - Rte_Call_<p>_<o>()

- What distinguishes a Standardized AUTOSAR interface from the other types?

  - It is both described using generic AUTOSAR means and its semantics are prescribed by the standard.

- Where are Standardized AUTOSAR interfaces exclusively used?

  - For services of the Basic Software (BSW).

- What kind of ports does the RTE use to connect with BSW?

  - Service Ports.

- What is the format of the API function used from SWC to RTE for a service call?

  - Rte_Call_<portPrototype>_<operation>()

- What is an example of a standardized operation in the Diagnostic Event Manager (DEM)?

  - SetEventStatus(In Dem_EventStatusType status)